# Lab 2 Report

System Design

The core time measurement was handled by TIM5, configured as a 32-bit free-running counter at 1 MHz. Each button press on B1 (PC13, falling edge EXTI) captured the current counter value. The time difference between presses was used to calculate BPM:

BPM= 60,000,000∆t

The beat period was scheduled using TIM6, which generated interrupts at the appropriate interval for the current BPM. In its ISR, the system started TIM15 in one-pulse PWM mode to produce a short (~20 ms) 2.5 kHz buzzer beep and triggered a visual LED “blip.”

For continuous breathing, the on-board LED (PA5, TIM2 Channel 1) was driven with PWM at ~1 kHz. TIM7 generated interrupts at 200 Hz to update LED duty cycle based on a cosine wave:

CCR1=0.5-0.5*cosphase*ARR

On each beat, the duty cycle was temporarily raised to ~80% to create a visible “blip.” After several ISR cycles, the LED returned to breathing mode.

Measurements

TIM5: Free-running counter at 1 MHz (PSC=79, ARR=0xFFFFFFFF).

TIM2: PWM for LED at ~1 kHz (PSC=79, ARR=999, CCR1 updated in breathing ISR).

TIM7: Breathing ISR at 200 Hz (PSC=799, ARR=499).

TIM15: One-pulse PWM for buzzer, ~2.5 kHz tone (PSC=79, ARR=319, CCR1=160) with ~20 ms duration.

TIM6: Beat scheduler; ARR updated dynamically from tap BPM (60000000 / Δt).

Improvements

Some changes to the original lab material were needed to achieve the desired results. TIM5 was used instead of TIM3 for the 1 MHz counter, requiring small prescaler adjustments but still meeting the timing needs. ARR/PSC values were also tuned on TIM2 and TIM15 to get stable LED PWM (~1 kHz) and buzzer tone (~2.5 kHz, ~20 ms).

Future improvements could include adding averaging over multiple taps for smoother BPM detection, expanding the valid BPM range, and refining the LED blip timing to make the beat more distinct. There could also potentially be improvements on the volume of the buzzer.

Video: Embedded Lab2